EET 3422: ANTENNA THEORY & PRACTICE

ASSIGNMENT 1

- Define the following terms as they relate to antennas (5 marks)

- Antenna efficiency

- Beam efficiency

- Antenna impedance

- Polarization

- Radiation intensity

- An antenna has a Half-Power Beam Width (HPBW) of 30° (5 marks)

- Calculate the First Null Beam Width (FNBW).

- If the power radiated in the main lobe is described by the equation P(θ) = P₀cos²(θ), where P₀ = 80 W, determine the angles at which the power first reaches 1% of the maximum power and when it reaches 10% of the maximum power.

- A transmission line with a characteristic impedance of Z0=75 Ω is connected to an antenna with an impedance ZA=50 Ω. The transmitter provides a power of 100 W. (6 marks)

- Calculate the reflection coefficient Γ.

- Determine the fraction of the power reflected towards the transmitter.

- How much power is delivered to the antenna?

- A narrowband antenna has a center frequency of 1 GHz and a relative bandwidth of 1%. (4 marks)

- Calculate the absolute bandwidth of the antenna.

- Determine the highest and lowest frequencies that the antenna can effectively transmit or receive.

- Explain in detail the concept of radiation mechanisms in antennas and briefly describe three conditions under which radiation can occur in antennas. (4 marks)

- An antenna radiates a maximum radiation intensity of 100 W/sr in its main lobe, while the total power radiated by the antenna is Prad=60 W. Calculate the directivity of the antenna and the beam area. (4 marks)

- Beam efficiency is crucial when choosing antenna efficiency. Briefly explain the conditions under which you will require an antenna with: (2 marks)

- Low beam efficiency

- High beam efficiency