EET3422: ANTENNA THEORY & PRACTICE

Lab 2: Horn Antenna

Design and Analyze Perforated Horn Antenna for RF Applications

Group: #

|  | Name | Registration Number (in order) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

SOFTWARE: MATLAB R2018a or later

This exercise shows how to design a perforated horn antenna suited for RF applications using custom geometry features of the Antenna Toolbox™ and analyze its scattering parameters. By following this guide, you will learn how to construct the antenna structure step by step, including the waveguide, horn taper, side grids, back grids, top and bottom grids, and finally, integrate feed mechanisms. This example is particularly useful for engineers and researchers focused on antenna design for high-frequency applications.

Define Geometric Parameters

The perforated horn antenna consists of a rectangular waveguide and a tapered horn with perforations. Define dimensions of the waveguide, tapered horn, and perforation in meters.

% Waveguide

waveguide_length = 10.16e-3;

waveguide_width = 22.86e-3;

waveguide_height = 31e-3;

% Horn Taper

flare_height = 0.01016;

flare_width = 0.02286;

flare_length = 0.053;

% Grid

grid_width = 0.002;

Create Waveguide

Start by creating the waveguide using a box shape. Then modify this shape by removing certain face and applying rotations and translations to achieve the desired orientation.

wShape = shape.Box(Length=waveguide_length,Width=waveguide_width,Height=waveguide_height);

removeFaces(wShape,2);

Paste your Figure here

rotate(wShape,90,[0 0 0],[0 1 0]);

Paste your Figure here

translate(wShape,[0 0 waveguide_length/2]);

Paste your Figure here

Create Tapered Horn Antenna Shape

The tapered horn is crucial for impedance matching and directing the wave propagation. Create the tapered horn antenna shape in three steps as below.

Create Horn

To create a tapered horn, extrude a rectangle, rotate it to align with the waveguide, and then modify its orientation.

rect1 = shape.Rectangle(Length=flare_height,Width=flare_width);

hornTaper1 = extrudeLinear(rect1,flare_length,Scale=[3.149 1.3998],NumSegments=1,Caps=true);

[~] = rotate(hornTaper1,90,[0 0 0],[0 1 0]);

[~] = translate(hornTaper1,[waveguide_height/2 0 flare_height/2]);

removeFaces(hornTaper1,3);

Paste your Figure here

Add Waveguide

Add waveguide to the tapered horn shape.

wShape = add(wShape,hornTaper1);

Add Perforations

Perforations in the form of a grid are added to tapered horn antenna to fine-tune its radiation pattern and bandwidth. Add various grids by subtracting box shapes from the main structure.

Create Side, Back, Top, and Bottom Grids

This section involves iterative subtraction to create a patterned structure on the antenna's surface.

Create the side grid.

xs=0.01200;

ys=0;

zs=0.00508;

topbox=shape.Box(Length=grid_width,width=26.86e-3,Height=0.009,Center=[xs ys zs]);

gapbc=4e-3;

for i=0:6

topbox.Center=[xs-i*gapbc 0 zs];

wShape=subtract(wShape,topbox);

end

Repeat similar steps for back, top, and bottom grids with specific dimensions and positions.

%Back grids

xba=-0.0155;

yba=0.008;

zba=0.00508;

topbox=shape.Box(Length=0.005,width=grid_width,Height=0.009,Center=[xba yba zba]);

gapbc=4e-3;

for i=0:4

topbox.Center=[xba yba-i*gapbc zs];

wShape=subtract(wShape,topbox);

end

%Top and Bottom grids

xb=12e-3;

yb=8e-3;

zb=11e-3;

gapbc=3.985e-3;

for i=0:6

topbox=shape.Box(Length=grid_width,width=0.002,Height=0.025,Center=[xb yb zb]);

if(i>3)

topbox.Height=0.005;

end

for j=0:4

topbox.Center=[xb-i*gapbc yb-j*gapbc zb];

wShape=subtract(wShape,topbox);

end

end

Finalize Antenna Structure

After integrating all components, including the waveguide and grids, finalize the antenna structure by adding holes of varying sizes to further refine its performance characteristics.

hornShape=wShape;

% Starting Position along x,y,z

xt=0.018;

yt=0;

zt=0.00;

w=0.002;

% Create a box and subtract from the shape to form grids.

topbox=shape.Box(Length=grid_width,width=w,Height=0.050,Center=[xt yt zt]);

gapbc=3.995e-3;

endp=11e-3;

endn=-11e-3;

incf = 182.8e-6;

Form holes along the flare length.

for i=0:18

temp=1e-3;

size=2e-3;

gapx=2e-3;

for k=1:2

if k==1

sign=1;

st=1;

else

sign=-1;

st=0;

end

for j=st:6

if(temp+j*size+j*gapx<=endp)

w=0.002;

yt=sign*j*gapbc;

elseif(temp+(j-1)*size+j*gapx<endp)

w=endp-temp-(j-1)*size-j*gapx;

yt=sign*(j-1)*gapbc+sign*(size/2+gapx+w/2);

else

break;

end

topbox.Width = w;

topbox.Center = [xt yt zt];

hornShape = subtract(hornShape,topbox);

end

end

yt = 0;

endp = endp+incf;

xt = xt+4e-3;

end

figure

show(hornShape);

Paste the result of the output here

Sequentially subtract box shapes to create holes in the antenna structure. Continue with subsequent holes, adjusting dimensions and positions as required to form holes along flare height.

for i=1:13

hole1 = shape.Box(Length=0.002, Width=42e-3, Height=0.006+(1.5e-3*(i-1)), Center=[0.018+(4e-3*(i-1)) 0 0.00508]);

hornShape = subtract(hornShape,hole1);

end

removeFaces(hornShape,5);

Paste the result of the output here

Integrate Feed and Analyze Antenna

The final steps involve integrating the feed mechanism and analyzing the antenna's scattering parameters over 8GHz - 12GHz frequency range.

Create a rectangular shape for the feed probe.

fo = -7.5e-3;

feedProbe1 = shape.Rectangle(Length=0.00508, Width=0.0014);

feedProbe = copy(feedProbe1);

Create the perforated horn antenna by assigning the custom perforated shape created in previous steps to the Shape property of the customAntenna object.

Get

feedProbe2 = shape.Rectangle(Length=0.0019, Width=0.0038);

[~] = translate(feedProbe2,[-feedProbe1.Length/2 0 0]);

feedProbe1 = feedProbe1+feedProbe2;

[~] = rotateY(feedProbe1,90);

translate(feedProbe1,[fo 0 feedProbe.Length/2]);

Paste the result of the output here

imp_rect = shape.Rectangle(Length=round(feedProbe.Width/sqrt(2),4),Width=round(feedProbe.Width/sqrt(2),4));

[~] = rotateZ(imp_rect,45);

[~] =translate(imp_rect,[fo 0 0]);

hornShape = imprintShape(hornShape,imp_rect);

antShape = add(hornShape,feedProbe1);

ant = customAntenna(Shape=antShape);

Add feed to the customAntenna using the createFeed function.

Get

[~] = createFeed(ant,[fo 0 0],1);

figure

show(ant)

Paste the result of the output here

figure

mesh(ant,MaxEdgeLength=0.01,MinEdgeLength=0.002);

Paste the result of the output here

Calculate the pattern of this antenna.

figure

pattern(ant,10e9);

Paste the result of the output here

Question

- Indicate the minimum and maximum pattern values in dBi.

Plot the directivity along the Azimuth angle

figure

patternAzimuth(ant,10e9,90);

Paste the result of the output here

Plot the directivity along the elevation angle

figure

patternElevation(ant,10e9);

Paste the result of the output here

Review Questions

- Indicate the following in the pattern of directivity

- Main lobe

- Side lobes

- Back lobes

- At which angle does the maximum radiation occur and indicate the value of directivity at this angle in dBi.